Alejandro Ossio, A01209122

Gerardo Naranjo, A01209499

Roberto Figueroa, A01209689

Computer Interfacing and Peripheral Equipment

Laboratory Exercise 03a

Rick L. Swenson

Lab Objective:

Understanding timing parameters, looking closely at glitches, measuring propagations delays

Material:

- TTL logic gates: 74LS04 (Inverter). 74LS08 (AND)

- CMOS logic gates: CD4069 (Totem-Pole Inverter).

- Function Generator, Power supply, Multimeter and Oscilloscope

Introduction:

The timing parameters commonly used in digital circuitry are:

tr: rise-time

ft: fall-time

tPLH: Propagation Delay Low to High

tPHL: Propagation Delay High to Low

δ: Average propagation delay

| Figure 1: Timing parameters | Average Propagation Delay |
| --- | --- |

Part 1: Measuring Timing Parameters

Assemble the circuit shown in Figure 2. Obtain and measure tr, ft, tPLH, tPHL and δ using an oscilloscope. First try with a TTL 74LS04 inverter and then with a CMOS CD4069.

Figure 2: Measuring Timing Parameters

74LS04:

tr=35 ns

tf=7 ns

tPLH= 5 ns

tPLH= 17 ns

δ=5 ns+17 ns2=11 ns

CD4069:

tr=87 ns

tr=48 ns

tPLH=25 ns

tPLH=53 ns

δ=25 ns+53 ns2=39 ns

Part 2: Looking closely for glitches.

Assemble the circuit shown in Figure 3. Obtain the timing diagram shown in Figure 3 (right)

| Figure 3: Looking for a Glitch | Timing diagram showing glitch |
| --- | --- |

Part 3: Ring Oscillator

Assemble the circuit shown in Figure 4. It is called a ring oscillator. It is a simple way of determining the average propagation delay for a digital circuit. Notice that an odd number of gates is required to make it oscillate at full frequency. Determine the average propagation delay δ by carefully taking measurements in the oscilloscope.

Figure 4: A ring Oscillator

f=31.5 MHz

average propagation delay= 13*f=10.582 ns